# AIORS + SvxLink Pre-Release Testing Procedure

Document revision: 1.2 Last updated: 13 September 2026

# Purpose

Use this procedure to build AIORS, the AIORS MQTT agent, and SvxLink from the current uncommitted Windows working trees, copy the signed test artifacts directly to the Raspberry Pi, install them without publishing a GitHub release, and verify the complete installation before committing the source changes.

The examples use:

Windows deployment repository: C:\Users\sound\Documents\GitHub\aiors_bsp\.git\aiors_deploy

Raspberry Pi account: aiors

Raspberry Pi address: 192.168.0.19

AIORS hardware version: 1.0

Example test version: deploy-test-v1.0.2-1

Increment the test version for each new package, for example deploy-test-v1.0.2-2, so old and new artifacts cannot be confused.

# Which Terminal To Use

This procedure uses two different terminals. Check the prompt before running every command:

PS C:\...>       Windows PowerShell on the Windows PC

aiors@aiors:~ $      Bash shell on the Raspberry Pi

Commands containing Windows paths such as .\dist\... must run in Windows PowerShell. Commands containing Linux paths such as /home/aiors/... run on the Pi. Do not type the PowerShell scp upload command at an aiors@aiors:~ $ prompt. If you are connected to the Pi when a Windows command is required, run exit first to return to Windows PowerShell.

# Important Safety Notes

A devcal transmitter test can activate the physical PTT and transmit RF. Use a dummy load or another safe test arrangement and comply with the applicable radio regulations.

Do not publish dirty test artifacts as a production release.

ALLOW_DIRTY_RELEASE=1 is only for local pre-release testing.

Keep the Minisign secret key outside every Git repository. Never copy it to the Raspberry Pi.

The installer verifies the Minisign signature and SHA-256 checksums even for a local test deployment.

# 1. Prepare Windows PowerShell

Run on: Windows PC in PowerShell (PS C:\...>).

Open PowerShell and move to the deployment repository:

Set-Location 'C:\Users\sound\Documents\GitHub\aiors_bsp\.git\aiors_deploy'

Allow scripts only in the current PowerShell process:

Set-ExecutionPolicy -Scope Process -ExecutionPolicy Bypass

Confirm that the release script and signing key exist:

Test-Path .\scripts\New-DeploymentRelease.ps1

Test-Path "$env:USERPROFILE\.config\aiors-deploy\minisign.key"

Both commands must return True.

# 2. Build The Test Deployment

Run on: Windows PC in PowerShell (PS C:\...>).

Build AIORS hardware 1.0 and 1.1, the MQTT agent, and SvxLink, then package and sign the combined deployment:

.\scripts\New-DeploymentRelease.ps1 `

-Version deploy-test-v1.0.2-1 `

-Rebuild `

-MinisignSecretKey "$env:USERPROFILE\.config\aiors-deploy\minisign.key"

Enter the Minisign secret-key password when requested. Do not add -CreateDraft; this test remains local.

The command must finish with:

Release assets ready in ...\dist\deploy-test-v1.0.2-1

# 3. Verify The Local Assets

Run on: Windows PC in PowerShell (PS C:\...>).

List the generated files:

Get-ChildItem ".\dist\deploy-test-v1.0.2-1"

Confirm these five files exist:

aiors-arm64.tar.gz

svxlink-arm64-rootfs.tar.gz

deployment-manifest.env

SHA256SUMS

SHA256SUMS.minisig

Inspect the deployment manifest:

Get-Content ".\dist\deploy-test-v1.0.2-1\deployment-manifest.env"

For an uncommitted test build, AIORS_SOURCE_DIRTY or SVXLINK_SOURCE_DIRTY may be 1. The architecture must be arm64, the AIORS payload layout must be 2, and the MQTT agent must be listed.

# 4. Copy The Test To The Raspberry Pi

Run the upload commands in this section on the Windows PC in PowerShell (PS C:\...>). If your prompt currently reads aiors@aiors:~ $, run exit before continuing.

Confirm that PowerShell is in the deployment repository and that the local release directory exists:

Set-Location 'C:\Users\sound\Documents\GitHub\aiors_bsp\.git\aiors_deploy'

Test-Path '.\dist\deploy-test-v1.0.2-1'

Test-Path must return True. If it returns False, stop and run sections 1 through 3 first. The scp source is on the Windows PC, not on the Pi.

Confirm SSH access:

ssh aiors@192.168.0.19 "uname -m; hostname"

The architecture must be aarch64.

Create the remote test directory:

ssh aiors@192.168.0.19 "mkdir -p /home/aiors/aiors-test"

Copy the complete release directory:

scp.exe -r "C:/Users/sound/Documents/GitHub/aiors_bsp/.git/aiors_deploy/dist/deploy-test-v1.0.2-1" aiors@192.168.0.19:/home/aiors/aiors-test/

Copy the current local installer. This is important when the installer itself contains uncommitted changes:

scp.exe "C:/Users/sound/Documents/GitHub/aiors_bsp/.git/aiors_deploy/install.sh" aiors@192.168.0.19:/home/aiors/aiors-test/install.sh

Connect to the Pi:

ssh aiors@192.168.0.19

After connecting, the prompt changes to aiors@aiors:~ $.

Run on: Raspberry Pi (aiors@aiors:~ $). Verify the copied files:

ls -lh /home/aiors/aiors-test/deploy-test-v1.0.2-1

ls -l /home/aiors/aiors-test/install.sh

# 5. Back Up And Review The Pi Configuration

Run on: Raspberry Pi (aiors@aiors:~ $).

Create an explicit backup of the locally maintained SvxLink configuration:

sudo cp /etc/svxlink/svxlink.conf \

"/etc/svxlink/svxlink.conf.pre-release.$(date +%Y%m%d_%H%M%S)"

The installer preserves a locally modified /etc/svxlink/svxlink.conf. Update the device names manually when upgrading from the former aliases:

sudo sed -i \

-e 's#alsa:aiors_usc_a#alsa:sound_usc_a#g' \

-e 's#alsa:aiors_usc_b#alsa:sound_usc_b#g' \

-e 's#/dev/hidraw-usc-a#/dev/hidraw_usc_a#g' \

-e 's#/dev/hidraw-usc-b#/dev/hidraw_usc_b#g' \

/etc/svxlink/svxlink.conf

Confirm the active transmitter and device settings:

grep -nE '^LOGICS=|^RX=|^TX=|^AUDIO_DEV=|^PTT_TYPE=|^HID_DEVICE=|^HID_PTT_PIN=' \

/etc/svxlink/svxlink.conf

Expected device naming:

Channel A audio: alsa:sound_usc_a

Channel B audio: alsa:sound_usc_b

Channel A HID:   /dev/hidraw_usc_a

Channel B HID:   /dev/hidraw_usc_b

PTT type:        Hidraw

Verify that the private FRN configuration remains present and do not display its secrets in shared logs:

sudo test -r /etc/svxlink/svxlink.d/ModuleFrn.conf && \

echo "Private FRN configuration present"

# 6. Install The Local Test Deployment

Run on: Raspberry Pi (aiors@aiors:~ $).

Make the copied installer executable:

chmod +x /home/aiors/aiors-test/install.sh

Install the signed local assets. ALLOW_DIRTY_RELEASE=1 is required because the source changes have not been committed:

DEPLOY_ASSET_DIR="/home/aiors/aiors-test/deploy-test-v1.0.2-1" \

DEPLOY_VERSION="deploy-test-v1.0.2-1" \

AIORS_HW_VERSION="1.0" \

ALLOW_DIRTY_RELEASE=1 \

bash "/home/aiors/aiors-test/install.sh" 2>&1 | \

tee "/home/aiors/aiors-test/deploy-test-v1.0.2-1-install.log"

The installation must finish with:

DONE: AIORS + SvxLink install script completed successfully.

Deployment completed

Installed deployment: deploy-test-v1.0.2-1

Warnings about hardware that requires a reboot may be temporary. Any signature, checksum, archive, architecture, missing-library, or payload-validation error is a failed test and must be investigated before continuing.

# 7. Reboot

Run on: Raspberry Pi (aiors@aiors:~ $).

sudo reboot

Wait for the Pi to return. The SSH session will close during reboot.

Run on: Windows PC in PowerShell (PS C:\...>). Reconnect:

ssh aiors@192.168.0.19

# 8. Verify Versions And Services

Run on: Raspberry Pi (aiors@aiors:~ $).

Confirm the architecture and installed versions:

uname -m

/usr/local/bin/aiorsctl version

/usr/bin/svxlink --version

/usr/local/sbin/aiors-mqtt-agent --version

Confirm the two radio services are enabled and active:

systemctl is-enabled aiorsd svxlink

systemctl is-active aiorsd svxlink

systemctl status aiorsd svxlink --no-pager -l

The MQTT agent is installed but should remain disabled until its broker and TLS configuration are intentionally enabled:

systemctl is-enabled aiors-mqtt-agent 2>/dev/null || true

systemctl is-active aiors-mqtt-agent 2>/dev/null || true

For the default test configuration, disabled and inactive are expected.

# 9. Verify Sound And HID Names

Run on: Raspberry Pi (aiors@aiors:~ $).

Check the physical sound cards and stable ALSA aliases:

aplay -l

arecord -l

aplay -L | grep -E '^sound_usc_[ab]'

Expected card IDs and PCM/control aliases:

SOUND_USC_A

SOUND_USC_B

sound_usc_a

sound_usc_b

Check the CM108 HID aliases and permissions:

ls -l /dev/hidraw_usc_a /dev/hidraw_usc_b

readlink -f /dev/hidraw_usc_a

readlink -f /dev/hidraw_usc_b

The links must resolve to two different /dev/hidrawN nodes. The underlying nodes should normally be root:audio with mode 0660.

Confirm that the SvxLink service account can write both PTT devices:

sudo -u svxlink test -w /dev/hidraw_usc_a && echo "HID A write access OK"

sudo -u svxlink test -w /dev/hidraw_usc_b && echo "HID B write access OK"

If aliases are missing, capture the actual topology:

for device in /dev/hidraw[0-9]*; do

echo "$device: $(udevadm info -q path -n "$device")"

done

# 10. Test CM108 PTT With Devcal

Run on: Raspberry Pi (aiors@aiors:~ $).

Make the transmitter RF-safe before continuing. Stop SvxLink so devcal can open the audio and HID devices:

sudo systemctl stop svxlink

Test channel A:

sudo -u svxlink devcal -t -f1000 -d2500 \

/etc/svxlink/svxlink.conf Tx1

The initialization must report a supported CM108-family chip. Press T to toggle PTT on, verify the physical channel A PTT signal, press T again to release it, and use Ctrl+C to exit.

Test channel B:

sudo -u svxlink devcal -t -f1000 -d2500 \

/etc/svxlink/svxlink.conf Tx2

Repeat the same brief PTT test for channel B. If the electrical polarity is reversed, review HID_PTT_PIN=GPIO4 versus HID_PTT_PIN=!GPIO4 before continuing.

Restart SvxLink after both tests:

sudo systemctl start svxlink

systemctl status svxlink --no-pager -l

# 11. Verify AIORS Diagnostics

Run on: Raspberry Pi (aiors@aiors:~ $).

Run the same commands through the SvxLink service account used by production:

sudo -u svxlink /usr/local/bin/aiorsctl get diagnostics all

sudo -u svxlink /usr/local/bin/aiorsctl get psu all

sudo -u svxlink /usr/local/bin/aiorsctl get cm108 status all

sudo -u svxlink /usr/local/bin/aiorsctl get sa818 status all

Verify the software version, hardware version, uptime, PSU states, CM108 states, SA818 states, ADC readings, environment readings, and overall status. Hardware faults must be explained before accepting the release.

# 12. Verify SvxLink And FRN

Run on: Raspberry Pi (aiors@aiors:~ $).

Inspect the startup log:

sudo tail -n 250 /var/log/svxlink

The successful initialization must include:

Starting logic: RepeaterLogic

Loading RX

Loading TX

Loading module "ModuleFrn"

Event handler script successfully loaded.

NOTICE: Initialization done. Starting main application.

Look for current errors and FRN activity:

sudo grep -E 'ERROR|WARNING|Module Frn|FRN|Initialization done' \

/var/log/svxlink | tail -n 150

Allow the configured FRN autostart timeout to expire. Confirm that ModuleFrn is activated and connects successfully. From the remote FRN RunCmd interface, test:

get psu a

get psu b

get psu all

Verify that the returned FRN messages contain the expected voltage, current, calculated power, PSU state, and fault state for the requested channel or channels.

# 13. Verify Statistics Files

Run on: Raspberry Pi (aiors@aiors:~ $).

Check the snapshot, persistent totals, and history:

sudo ls -lh \

/var/lib/svxlink/svxstats_snapshot.txt \

/var/lib/svxlink/svxstats_totals.txt \

/var/lib/svxlink/svxstats_history.log

Inspect the latest snapshot:

sudo sed -n '1,240p' /var/lib/svxlink/svxstats_snapshot.txt

Verify that it contains AIORS and SvxLink versions, Linux/SvxLink/AIORS uptime, FRN state and counters, PSU measurements and energy totals, environmental and bus measurements, RF measurements, and CM108/SA818 current and historical fault information.

# 14. Inspect Current-Boot Logs

Run on: Raspberry Pi (aiors@aiors:~ $).

sudo journalctl -b -u aiorsd -u svxlink --no-pager -n 300

sudo journalctl -b -u aiors-mqtt-agent --no-pager -n 100

No service should be in a restart loop. MQTT being disabled must not prevent AIORS or SvxLink from starting.

Record useful status information in the test directory:

mkdir -p "/home/aiors/aiors-test/results/deploy-test-v1.0.2-1"

systemctl status aiorsd svxlink --no-pager -l \

> "/home/aiors/aiors-test/results/deploy-test-v1.0.2-1/service-status.txt"

sudo journalctl -b -u aiorsd -u svxlink --no-pager \

> "/home/aiors/aiors-test/results/deploy-test-v1.0.2-1/radio-services-journal.txt"

sudo cp /var/log/svxlink \

"/home/aiors/aiors-test/results/deploy-test-v1.0.2-1/svxlink.log"

sudo cp /var/lib/svxlink/svxstats_snapshot.txt \

"/home/aiors/aiors-test/results/deploy-test-v1.0.2-1/svxstats_snapshot.txt"

sudo chown -R aiors:aiors "/home/aiors/aiors-test/results/deploy-test-v1.0.2-1"

# 15. Optional Network-Outage Test

Run on: Raspberry Pi (aiors@aiors:~ $) using local console access.

Perform this only when local access to the Pi is available because SSH will be lost. Disconnect Ethernet briefly, reconnect it, and then confirm:

systemctl is-active aiorsd svxlink

systemctl show svxlink -p NRestarts -p ExecMainStartTimestamp

sudo tail -n 150 /var/log/svxlink

AIORS and SvxLink must remain operational. FRN should reconnect using its own network recovery. When MQTT is later enabled, the MQTT agent must independently reconnect and drain its queued telemetry without restarting either radio service.

# 16. Acceptance Checklist

The test passes only when all applicable items are true:

The package builds and signs without errors.

The Pi verifies the signature and every SHA-256 checksum.

The deployment completes without payload or unresolved-library errors.

AIORS and SvxLink are enabled and active after reboot.

The installed versions match the test build.

SOUND_USC_A/B and sound_usc_a/b are present.

/dev/hidraw_usc_a/b exist, map to different devices, and are writable by svxlink.

devcal detects both CM108 devices and controls both physical PTT outputs.

AIORS diagnostics return successfully and hardware faults are understood.

SvxLink reaches Initialization done without unexpected errors.

ModuleFrn activates, connects, and responds to all three PSU RunCmd requests.

Snapshot, totals, and history files are being updated.

MQTT remains isolated and cannot stop AIORS or SvxLink.

No service is crash-looping and no unexplained errors remain in the logs.

# 17. After The Test Passes

Run on: Windows PC in PowerShell (PS C:\...>).

Commit and push the reviewed changes in the AIORS, SvxLink, and aiors_deploy repositories. Confirm each working tree is clean:

git status --short

Build a new production version from the committed trees, for example:

Set-Location 'C:\Users\sound\Documents\GitHub\aiors_bsp\.git\aiors_deploy'

.\scripts\New-DeploymentRelease.ps1 `

-Version deploy-v1.0.2 `

-Rebuild `

-MinisignSecretKey "$env:USERPROFILE\.config\aiors-deploy\minisign.key" `

-CreateDraft

Review deployment-manifest.env. Both source dirty fields must be 0. Review the GitHub draft assets and release notes, then publish the release. Production Pi installations must omit ALLOW_DIRTY_RELEASE=1.

# 18. If The Test Fails

Keep the test package and captured logs. Record the failing command and exact output.

Run these checks on: Raspberry Pi (aiors@aiors:~ $).

systemctl status aiorsd svxlink --no-pager -l

sudo journalctl -b -u aiorsd -u svxlink --no-pager -n 300

sudo tail -n 250 /var/log/svxlink

ls -l /dev/hidraw* /dev/snd/* 2>/dev/null

Correct the source or deployment scripts on Windows, increment the test version, rebuild, copy the new directory, and repeat this procedure. Do not publish the release until the acceptance checklist passes.